Password Protected Universal Locking System

FOR THE FULFILMENT OF CARRICULUM OF BACHELOR OF TECHNOLOGY ON ELECTRONICS AND COMMUNICATION ENGINEERING OF WEST BENGAL UNIVERSITY OF TECHNOLOGY

BY

ABHINAV KUMAR (09104003002)

ABHIROOP MUKHERJEE (09104003005)

AVINASH KUMAR (09104003023)

SAGNIK HAJRA (09104003058)

SAGOR KUMAR GHOSH (09104003058)

SHUVAGATA BHATTACHARJEE (09104003070)

(ELECTRONICS AND COMMUNICATION ENGINEERING)

OF

INSTITUTE OF ENGINEERING AND MANAGEMENT

(SALTLAKE ELECTRONICS COMPLEX, CALCUTTA-700091, INDIA)

UNDER THE GUIDANCE OF

Mr ARUNAVA MUKHERJEE

(Asst professor, Dept of ECE, IEM)

ACKNOWLEDGEMENT

I have taken efforts in this project. However, it would not have been possible without the kind support and help of THE GROUP MEMBERS. I would like to extend my sincere thanks to all of them.

I am highly indebted to PROF. ARUNAVA MUKHERJEE for his guidance and constant supervision as well as for providing necessary information regarding the project & also PROF TEJENDRANATH SEN for his whole-hearted help in completing the project.

I would like to express my gratitude towards my parents &many other professors of INSTITUTE Of ENGINEERING AND MANAGEMENTfor their kind co-operation and encouragement which help me in completion of this project.

I would like to express my special gratitude and thanks to industry persons for giving me such attention and time.

My thanks and appreciations also go to some people who have willingly helped me out with their abilities.

TO ALL OF YOU THANK YOU VERY MUCH.

ABSTRACT

The microcontroller based digital lock is an access control system that allows only authorized persons to access any restricted division. The major components include a keypad, LCD, and the micro controller 89S52 .

The system is fully The AT89S52 which is a low-power, high-performance CMOS 8-bit microcontroller with 8Kbytes of in-system programmable Flash memory. The electronic control assembly allows the system to unlock the device with a password. A four digit predefined password needs to be specified the user. A 4x4 matrix KEYPAD and a 16x2 LCD have been used here to set the password which is stored so that we can change it at any time. While unlocking, if the entered password from keypad matches with the stored password, then the lock opens and a message is displayed on LCD. Also an output pin is made low to be used for further purpose. As the program starts, string ‘Enter Password’ is displayed on LCD. The keypad is scanned for pressed digits one by one. Every time, row and column of the key pressed is detected

. If all the digits match with set password, LCD displays ‘password is correct’ and the lock output pin goes high and the led glows. If the security code is wrong, ‘WARNING’ is sent to be displayed on LCD and if wrong password is input for three consecutive times ‘PROHIBITED’ is displayed on lcd and the buzzer rings which is connected to the microcontroller. It has wide applications in the present world. It is mainly used for door alarms, equipment privacy locks, cell phones,  computers and in many lock systems.

INDEX

CONTENTS

TOPIC                                                                       PAGE NO.

1 .Introduction

2.  Instruments Used

3.  MCU AT89s52

4.  HD44780(LCD Display)

5. Developments and implementations

6. Three main parts of project

A) Circuit Design

B) Dataflow

C)Source Code/Program

D) Door Lock Sysytem

7. Applications

8. Advantages & Disadvantages

9. Problems faced during development and solutions

10.Bibliography

ANNEXTURES

- Manual for Administrator

- Manual for User

INTRODUCTION

Security is a prime concern in our day to day life.Everyone wants to be as much secure as possible.An access control for doors forms a vital link in a security chain.The Microcontroller based door locking system is an access control system that allows only authorised persons to access a restricted area.

Password based universal locking system using Microcontroller is used in the places where security is a prime concern.It can be also used tosecure lockers and other protective doors.The system comprises of a number keypad and the keypads are connected to the Microcontroller89S52.This is one of the popular Microcontroller.it has got 40 pins.The Microcontroller has got a program memory of 2 kilobytes.The Microcontroller continuously monitors the keypad and if somebody enters the wrong password it will check the entered password with the password which was stored in the memory and if the password is same then the Microcontroller will switch on the corresponding device.If the password entered is wrong then a yellow LED will blink which is a warning sign for the user.Finally if the password entered is wrong more than three times then a RED LED will be ON which means the Alarm will be turned on, i e the Access is denied.Apart from this the Locking system is very flexible.The password already saved can be again RESET by the user.Additionally , it has the concept of Master password in which the administrator has got the sole authority over the Locking system.

INSTRUMENTS

- MICROCONTROLLER 89S52

- LCD BASE

- 4X4 MATRIX TYPE SWITCH

- PRINTED CIRCUIT BOARD

- LIQUID CRYSTAL DISPLAY

- RESISTORS

- CAPACITORS

- CRYSTAL OSCILLATOR

- CONNECTORS

- LIGHT EMITTING DIODE

- MICROCONTROLLER BASE

- ALARM

- +5V SUPPLY

- SOLDERING IRON

- CONTINUITY TESTER

- TACT SWITCH

- DIGITAL MULTIMETER

- SOLDERING FLUX & ALLOY

- CUTTERS & TWISORS

- BREADBOARD

- CONNECTING WIRES

MICROCONTROLLER AT89S52

FEATURES

- Compatible with MCS®-51 Products

- 8K Bytes of In-System Programmable (ISP) Flash Memory – Endurance: 10,000 Write/Erase Cycles

- 4.0V to 5.5V Operating Range

- Fully Static Operation: 0 Hz to 33 MHz

- Three-level Program Memory Lock

- 256 x 8-bit Internal RAM

- 32 Programmable I/O Lines

- Three 16-bit Timer/Counters

- Eight Interrupt Sources

- Full Duplex UART Serial Channel

- Low-power Idle and Power-down Modes

- Interrupt Recovery from Power-down Mode

- Watchdog Timer

- Dual Data Pointer

- Power-off Flag

- Fast Programming Time

- Flexible ISP Programming (Byte and Page Mode)

- Green (Pb/Halide-free) Packaging.

- DESCRIPTION

The AT89S52 is a low-power, high-performance CMOS 8-bit microcontroller with 8K bytes of in-system programmable Flash memory. The device is manufactured using Atmel’s high-density non-volatile memory technology and is compatible with the industry-standard 80C51 instruction set and pin out. The on-chip Flash allows the program memory to be reprogrammed in-system or by a conventional non-volatile memory programmer. By combining a versatile 8-bit CPU with in-system programmable Flash on a monolithic chip, the Atmel AT89S52 is a powerful microcontroller which provides a highly-flexible and cost-effective solution to many embedded control applications. The AT89S52 provides the following standard features: 8K bytes of Flash, 256 bytes of RAM, 32 I/O lines, Watchdog timer, two data pointers, three 16-bit timer/counters, a six-vector two-level interrupt architecture, a full duplex serial port, on-chip oscillator, and clock circuitry. In addition, the AT89S52 is designed with static logic for operation down to zero frequency and supports two software selectable power saving modes. The Idle Mode stops the CPU while allowing the RAM, timer/counters, serial port, and interrupt system to continue functioning. The Power-down mode saves the RAM con-tents but freezes the oscillator, disabling all other chip functions until the next interrupt or hardware reset.

Pin Description

VCC Supply voltage.

GND Ground.

Port 0

Port 0 is an 8-bit open drain bidirectional I/O port. As an output port, each pin can sink eight TTL inputs. When 1s are written to port 0 pins, the pins can be used as high-impedance inputs. Port 0 can also be configured to be the multiplexed low-order address/data bus during accesses to external program and data memory. In this mode, P0 has internal pull-ups. Port 0 also receives the code bytes during Flash programming and outputs the code bytes during program verification. External pull-ups are required during program verification.

Port 1

Port 1 is an 8-bit bidirectional I/O port with internal pull-ups. The Port 1 output buffers can sink/source four TTL inputs. When 1s are written to Port 1 pins, they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 1 pins that are externally being pulled low will source current (IIL) because of the internal pull-ups. In addition, P1.0 and P1.1 can be configured to be the timer/counter 2 external count input (P1.0/T2) and the timer/counter 2 trigger input (P1.1/T2EX), respectively, as shown in the following table. Port 1 also receives the low order address bytes during Flash programming and verification.

Port 2

Port 2 is an 8-bit bidirectional I/O port with internal pull-ups. The Port 2 output buffers can sink/source four TTL inputs. When 1s are written to Port 2 pins, they are pulled high by the inter-nal pull-ups and can be used as inputs. As inputs, Port 2 pins that are externally being pulled low will source current (IIL) because of the internal pull-ups. Port 2 emits the high-order address byte during fetches from external program memory and during accesses to external data memory that use 16-bit addresses (MOVX @ DPTR). In this application, Port 2 uses strong internal pull-ups when emitting 1s. During accesses to external data memory that use 8-bit addresses (MOVX @ RI), Port 2 emits the contents of the P2 Special Function Register. Port 2 also receives the high-order address bits and some control signals during Flash programming and verification.

Port 3

Port 3 is an 8-bit bidirectional I/O port with internal pull-ups. The Port 3 output buffers can sink/source four TTL inputs. When 1s are written to Port 3 pins, they are pulled high by the internal pull-ups and can be used as inputs. As inputs, Port 3 pins that are externally being pulled low will source current (IIL) because of the pull-ups. Port 3 receives some control signals for Flash programming and verification. Port 3 also serves the functions of various special features of the AT89S52, as shown in the following table.

RST

Reset input. A high on this pin for two machine cycles while the oscillator is running resets the device. This pin drives high for 98 oscillator periods after the Watchdog times out. The DISRTO bit in SFR AUXR (address 8EH) can be used to disable this feature. In the default state of bit DISRTO, the RESET HIGH out feature is enabled.

ALE/PROG

Address Latch Enable (ALE) is an output pulse for latching the low byte of the address during accesses to external memory. This pin is also the program pulse input (PROG) during Flash programming. In normal operation, ALE is emitted at a constant rate of 1/6 the oscillator frequency and may be used for external timing or clocking purposes. Note, however, that one ALE pulse is skipped during each access to external data memory. If desired, ALE operation can be disabled by setting bit 0 of SFR location 8EH. With the bit set, ALE is active only during a MOVX or MOVC instruction. Otherwise, the pin is weakly pulled high. Setting the ALE-disable bit has no effect if the microcontroller is in external execution mode.

PSEN

Program Store Enable (PSEN) is the read strobe to external program memory. When the AT89S52 is executing code from external program memory, PSEN is activated twice each machine cycle, except that two PSEN activations are skipped during each access to external data memory.

EA/VPP

External Access Enable. EA must be strapped to GND in order to enable the device to fetch code from external program memory locations starting at 0000H up to FFFFH. Note, however, that if lock bit 1 is programmed, EA will be internally latched on reset. EA should be strapped to VCC for internal program executions. This pin also receives the 12-volt programming enable voltage (VPP) during Flash programming.

XTAL1

Input to the inverting oscillator amplifier and input to the internal clock operating circuit.

XTAL2

Output from the inverting oscillator amplifier.

Special Function Registers

A map of the on-chip memory area called the Special Function Register (SFR) space is shown in Table 5-1. Note that not all of the addresses are occupied, and unoccupied addresses may not be implemented on the chip. Read accesses to these addresses will in general return random data, and write accesses will have an indeterminate effect. User software should not write 1s to these unlisted locations, since they may be used in future products to invoke new features. In that case, the reset or inactive values of the new bits will always be 0.

Timer 2 Registers:

Control and status bits are contained in registers T2CON (shown in Table 5- 2) and T2MOD (shown in Table 10-2) for Timer 2. The register pair (RCAP2H, RCAP2L) are the Capture/Reload registers for Timer 2 in 16-bit capture mode or 16-bit auto-reload mode.

Interrupt Registers:

The individual interrupt enable bits are in the IE register. Two priorities can be set for each of the six interrupt sources in the IP register.

Memory Organization

MCS-51 devices have a separate address space for Program and Data Memory. Up to 64K bytes each of external Program and Data Memory can be addressed.

Program Memory

If the EA pin is connected to GND, all program fetches are directed to external memory. On the AT89S52, if EA is connected to VCC, program fetches to addresses 0000H through 1FFFH are directed to internal memory and fetches to addresses 2000H through FFFFH are to external memory.

Data Memory

The AT89S52 implements 256 bytes of on-chip RAM. The upper 128 bytes occupy a parallel address space to the Special Function Registers. This means that the upper 128 bytes have the same addresses as the SFR space but are physically separate from SFR space. When an instruction accesses an internal location above address 7FH, the address mode used in the instruction specifies whether the CPU accesses the upper 128 bytes of RAM or the SFR space. Instructions which use direct addressing access the SFR space. For example, the following direct addressing instruction accesses the SFR at location 0A0H (which is P2). MOV 0A0H, #data Instructions that use indirect addressing access the upper 128 bytes of RAM. For example, the following indirect addressing instruction, where R0 contains 0A0H, accesses the data byte at address 0A0H, rather than P2 (whose address is 0A0H). MOV @R0, #data Note that stack operations are examples of indirect addressing, so the upper 128 bytes of data RAM are available as stack space.

Watchdog Timer (One-time Enabled with Reset-out)

The WDT is intended as a recovery method in situations where the CPU may be subjected to software upsets. The WDT consists of a 14-bit counter and the Watchdog Timer Reset (WDTRST) SFR. The WDT is defaulted to disable from exiting reset. To enable the WDT, a user must write 01EH and 0E1H in sequence to the WDTRST register (SFR location 0A6H). When the WDT is enabled, it will increment every machine cycle while the oscillator is running. The WDT timeout period is dependent on the external clock frequency. There is no way to disable the WDT except through reset (either hardware reset or WDT overflow reset). When WDT over-flows, it will drive an output RESET HIGH pulse at the RST pin.

Using the WDT

To enable the WDT, a user must write 01EH and 0E1H in sequence to the WDTRST register (SFR location 0A6H). When the WDT is enabled, the user needs to service it by writing 01EH and 0E1H to WDTRST to avoid a WDT overflow. The 14-bit counter overflows when it reaches 16383 (3FFFH), and this will reset the device. When the WDT is enabled, it will increment every machine cycle while the oscillator is running. This means the user must reset the WDT at least every 16383 machine cycles. To reset the WDT the user must write 01EH and 0E1H to WDTRST. WDTRST is a write-only register. The WDT counter cannot be read or written. When upon exit from IDLE.

UART

The UART in the AT89S52 operates the same way as the UART in the AT89C51 and AT89C52. For further information on the UART operation, please click on the document link below:

Timer 0 and 1

Timer 0 and Timer 1 in the AT89S52 operate the same way as Timer 0 and Timer 1 in the AT89C51 and AT89C52

Capture Mode

In the capture mode, two options are selected by bit EXEN2 in T2CON. If EXEN2 = 0, Timer 2 is a 16-bit timer or counter which upon overflow sets bit TF2 in T2CON. This bit can then be used to generate an interrupt. If EXEN2 = 1, Timer 2 performs the same operation, but a 1-to-0 transition at external input T2EX also causes the current value in TH2 and TL2 to be captured into RCAP2H and RCAP2L, respectively. In addition, the transition at T2EX causes bit EXF2 in T2CON to be set. The EXF2 bit, like TF2, can generate an interrupt.

Auto-reload (Up or Down Counter)

Timer 2 can be programmed to count up or down when configured in its 16-bit auto-reload mode. This feature is invoked by the DCEN (Down Counter Enable) bit located in the SFR T2MOD (see Table 10-2). Upon reset, the DCEN bit is set to 0 so that timer 2 will default to count up. When DCEN is set, Timer 2 can count up or down, depending on the value of the T2EX pin.

Timer 2

Timer 2 is a 16-bit Timer/Counter that can operate as either a timer or an event counter. The type of operation is selected by bit C/T2 in the SFR T2CON (shown in Table 5-2). Timer 2 has three operating modes: capture, auto-reload (up or down counting), and baud rate generator. The modes are selected by bits in T2CON, as shown in Table 10-1. Timer 2 consists of two 8-bit registers, TH2 and TL2. In the Timer function, the TL2 register is incremented every machine cycle. Since a machine cycle consists of 12 oscillator periods, the count rate is 1/12 of the oscil-lator frequency.

In the Counter function, the register is incremented in response to a 1-to-0 transition at its corresponding external input pin, T2. In this function, the external input is sampled during S5P2 of every machine cycle. When the samples show a high in one cycle and a low in the next cycle, the count is incremented. The new count value appears in the register during S3P1 of the cycle following the one in which the transition was detected. Since two machine cycles (24 oscillator periods) are required to recognize a 1-to-0 transition, the maximum count rate is 1/24 of the oscillator frequency. To ensure that a given level is sampled at least once before it changes, the level should be held for at least one full machine cycle.

Figure 10-2 shows Timer 2 automatically counting up when DCEN = 0. In this mode, two options are selected by bit EXEN2 in T2CON. If EXEN2 = 0, Timer 2 counts up to 0FFFFH and then sets the TF2 bit upon overflow. The overflow also causes the timer registers to be reloaded with the 16-bit value in RCAP2H and RCAP2L. The values in Timer in Capture ModeRCAP2H and RCAP2L are pre-set by software. If EXEN2 = 1, a 16-bit reload can be triggered either by an overflow or by a 1-to-0 transition at external input T2EX. This transition also sets the EXF2 bit. Both the TF2 and EXF2 bits can generate an interrupt if enabled. Setting the DCEN bit enables Timer 2 to count up or down, as shown in Figure 10-2. In this mode, the T2EX pin controls the direction of the count. A logic 1 at T2EX makes Timer 2 count up. The timer will overflow at 0FFFFH and set the TF2 bit. This overflow also causes the 16-bit value in RCAP2H and RCAP2L to be reloaded into the timer registers, TH2 and TL2, respectively. A logic 0 at T2EX makes Timer 2 count down. The timer underflows when TH2 and TL2 equal the values stored in RCAP2H and RCAP2L. The underflow sets the TF2 bit and causes 0FFFFH to be reloaded into the timer registers. The EXF2 bit toggles whenever Timer 2 overflows or underflows and can be used as a 17th bit of resolution. In this operating mode, EXF2 does not flag an interrupt.

Baud Rate Generator

Timer 2 is selected as the baud rate generator by setting TCLK and/or RCLK in T2CON (Table 5-2). Note that the baud rates for transmit and receive can be different if Timer 2 is used for the receiver or transmitter and Timer 1 is used for the other function. Setting RCLK and/or TCLK puts Timer 2 into its baud rate generator mode, as shown in Figure 11-1. The baud rate generator mode is similar to the auto-reload mode, in that a rollover in TH2 causes the Timer 2 registers to be reloaded with the 16-bit value in registers RCAP2H and RCAP2L, which are pre-set by software. The baud rates in Modes 1 and 3 are determined by Timer 2’s overflow rate according to the following equation.

Modes 1 and 3 Baud Rates = Time 2 overflow rate / 16

The Timer can be configured for either timer or counter operation. In most applications, it is con-figured for timer operation (CP/T2 = 0). The timer operation is different for Timer 2 when it is used as a baud rate generator. Normally, as a timer, it increments every machine cycle (at 1/12 the oscillator frequency). As a baud rate generator, however, it increments every state time (at 1/2 the oscillator frequency). The baud rate formula is given below.

- Modes 1 and 3/ Baud Rate = oscillator frequency/(32*{65536-RCAP2H,RCAP2L)

- Where (RCAP2H, RCAP2L) is the content of RCAP2H and RCAP2L taken as a 16-bit unsigned integer. Timer 2 as a baud rate generator is shown in Figure 11-1. This figure is valid only if RCLK or TCLK = 1 in T2CON. Note that a rollover in TH2 does not set TF2 and will not generate an inter-rupt. Note too, that if EXEN2 is set, a 1-to-0 transition in T2EX will set EXF2 but will not cause a reload from (RCAP2H, RCAP2L) to (TH2, TL2). Thus, when Timer 2 is in use as a baud rate generator, T2EX can be used as an extra external interrupt.

Note that when Timer 2 is running (TR2 = 1) as a timer in the baud rate generator mode, TH2 or TL2 should not be read from or written to. Under these conditions, the Timer is incremented every state time, and the results of a read or write may not be accurate. The RCAP2 registers may be read but should not be written to, because a write might overlap a reload and cause write and/or reload errors. The timer should be turned off (clear TR2) before accessing the Timer 2 or RCAP2 registers.

Programmable Clock Out

A 50% duty cycle clock can be programmed to come out on P1.0, as shown in Figure 12-1. This pin, besides being a regular I/O pin, has two alternate functions. It can be programmed to input the external clock for Timer/Counter 2 or to output a 50% duty cycle clock ranging from 61 Hz to 4 MHz (for a 16-MHz operating frequency). To configure the Timer/Counter 2 as a clock generator, bit C/T2 (T2CON.1) must be cleared and bit T2OE (T2MOD.1) must be set. Bit TR2 (T2CON.2) starts and stops the timer. The clock-out frequency depends on the oscillator frequency and the reload value of Timer 2 capture registers (RCAP2H, RCAP2L), as shown in the following equation.

Clock-Out Frequency = Oscillator Frequency /4 x [65536-(RCAP2H, RCAP2L)]

In the clock-out mode, Timer 2 roll-overs will not generate an interrupt. This behaviour is similar to when Timer 2 is used as a baud-rate generator. It is possible to use Timer 2 as a baud-rate generator and a clock generator simultaneously. Note, however, that the baud-rate and clock-out frequencies cannot be determined independently from one another since they both use RCAP2H and RCAP2L.

Interrupts

The AT89S52 has a total of six interrupt vectors: two external interrupts (INT0 and INT1), three timer interrupts (Timers 0, 1, and 2), and the serial port interrupt. These interrupts are all shown in Figure 13-1. Each of these interrupt sources can be individually enabled or disabled by setting or clearing a bit in Special Function Register IE. IE also contains a global disable bit, EA, which disables all interrupts at once. Note that Table 13-1 shows that bit position IE.6 is unimplemented. User software should not write a 1 to this bit position, since it may be used in future AT89 products. Timer 2 interrupt is generated by the logical OR of bits TF2 and EXF2 in register T2CON. Neither of these flags is cleared by hardware when the service routine is vectored to. In fact, the service routine may have to determine whether it was TF2 or EXF2 that generated the interrupt, and that bit will have to be cleared in software. The Timer 0 and Timer 1 flags, TF0 and TF1, are set at S5P2 of the cycle in which the timers overflow. The values are then polled by the circuitry in the next cycle. However, the Timer 2 flag, TF2, is set at S2P2 and is polled in the same cycle in which the timer overflows.

14. Oscillator Characteristics

XTAL1 and XTAL2 are the input and output, respectively, of an inverting amplifier that can be configured for use as an on-chip oscillator, as shown in Figure 16-1. Either a quartz crystal or ceramic resonator may be used. To drive the device from an external clock source, XTAL2 should be left unconnected while XTAL1 is driven, as shown in Figure 16-2. There are no requirements on the duty cycle of the external clock signal, since the input to the internal clocking circuitry is through a divide-by-two flip-flop, but minimum and maximum voltage high and low time specifications must be observed.

15. Idle Mode

In idle mode, the CPU puts itself to sleep while all the on-chip peripherals remain active. The mode is invoked by software. The content of the on-chip RAM and all the special functions registers remain unchanged during this mode. The idle mode can be terminated by any enabled interrupt or by a hardware reset. Note that when idle mode is terminated by a hardware reset, the device normally resumes pro-gram execution from where it left off, up to two machine cycles before the internal reset algorithm takes control. On-chip hardware inhibits access to internal RAM in this event, but access to the port pins is not inhibited. To eliminate the possibility of an unexpected write to a port pin when idle mode is terminated by a reset, the instruction following the one that invokes idle mode should not write to a port pin or to external memory

16. Power-down Mode

In the Power-down mode, the oscillator is stopped, and the instruction that invokes Power-down is the last instruction executed. The on-chip RAM and Special Function Registers retain their values until the Power-down mode is terminated. Exit from Power-down mode can be initiated either by a hardware reset or by an enabled external interrupt. Reset redefines the SFRs but does not change the on-chip RAM. The reset should not be activated before VCC is restored to its normal operating level and must be held active long enough to allow the oscillator to restart.

18. Programming the Flash – Parallel Mode

The AT89S52 is shipped with the on-chip Flash memory array ready to be programmed. The programming interface needs a high-voltage (12-volt) program enable signal and is compatible with conventional third-party Flash or EPROM programmers. The AT89S52 code memory array is programmed byte-by-byte.

Programming Algorithm:

Before programming the AT89S52, the address, data, and control signals should be set up according to the “Flash Programming Modes” (Table 22-1) and Figure 22-1 and Figure 22-2. To program the AT89S52, take the following steps: 1. Input the desired memory location on the address lines. 2. Input the appropriate data byte on the data lines. 3. Activate the correct combination of control signals. 4. Raise EA/VPP to 12V. 5. Pulse ALE/PROG once to program a byte in the Flash array or the lock bits. The byte-write cycle is self-timed and typically takes no more than 50 μs. Repeat steps 1 through 5, changing the address and data for the entire array or until the end of the object file is reached.

Data Polling:

The AT89S52 features Data Polling to indicate the end of a byte write cycle. During a write cycle, an attempted read of the last byte written will result in the complement of the written data on P0.7. Once the write cycle has been completed, true data is valid on all outputs, and the next cycle may begin. Data Polling may begin any time after a write cycle has been initiated.

Ready/Busy:

The progress of byte programming can also be monitored by the RDY/BSY output signal. P3.0 is pulled low after ALE goes high during programming to indicate BUSY. P3.0 is pulled high again when programming is done to indicate READY.

Program Verify:

If lock bits LB1 and LB2 have not been programmed, the programmed code data can be read back via the address and data lines for verification. The status of the individual lock bits can be verified directly by reading them back.

Reading the Signature Bytes:

The signature bytes are read by the same procedure as a nor-mal verification of locations 000H, 100H, and 200H, except that P3.6 and P3.7 must be pulled to a logic low. The values returned are as follows. (000H) = 1EH indicates manufactured by Atmel (100H) = 52H indicates AT89S52 (200H) = 06H

Chip Erase:

In the parallel programming mode, a chip erase operation is initiated by using the proper combination of control signals and by pulsing ALE/PROG low for a duration of 200 ns - 500 ns. In the serial programming mode, a chip erase operation is initiated by issuing the Chip Erase instruction. In this mode, chip erase is self-timed and takes about 500 ms. During chip erase, a serial read from any address location will return 00H at the data output.

19. Programming the Flash – Serial Mode

The Code memory array can be programmed using the serial ISP interface while RST is pulled to VCC. The serial interface consists of pins SCK, MOSI (input) and MISO (output). After RST is set high, the Programming Enable instruction needs to be executed first before other operations can be executed. Before a reprogramming sequence can occur, a Chip Erase operation is required. The Chip Erase operation turns the content of every memory location in the Code array into FFH. Either an external system clock can be supplied at pin XTAL1 or a crystal needs to be connected across pins XTAL1 and XTAL2. The maximum serial clock (SCK) frequency should be less than 1/16 of the crystal frequency. With a 33 MHz oscillator clock, the maximum SCK frequency is 2MHZ.

20. Serial Programming Algorithm

To program and verify the AT89S52 in the serial programming mode, the following sequence is recommended: 1. Power-up sequence: a. Apply power between VCC and GND pins. b. Set RST pin to “H”. If a crystal is not connected across pins XTAL1 and XTAL2, apply a 3 MHz to 33 MHz clock to XTAL1 pin and wait for at least 10 milliseconds. 2. Enable serial programming by sending the Programming Enable serial instruction to pin MOSI/P1.5. The frequency of the shift clock supplied at pin SCK/P1.7 needs to be less than the CPU clock at XTAL1 divided by 16. 3. The Code array is programmed one byte at a time in either the Byte or Page mode. The write cycle is self-timed and typically takes less than 0.5 ms at 5V. 4. Any memory location can be verified by using the Read instruction which returns the content at the selected address at serial output MISO/P1.6. 5. At the end of a programming session, RST can be set low to commence normal device operation.

Power-off sequence (if needed):

1. Set XTAL1 to “L” (if a crystal is not used).

2. Set RST to “L”.

3. Turn VCC power off.

Data Polling:

The Data Polling feature is also available in the serial mode. In this mode, during a write cycle an attempted read of the last byte written will result in the complement of the MSB of the serial output byte on MISO.

21. Serial Programming Instruction Set

The Instruction Set for Serial Programming follows a 4-byte protocol and is shown in Table 24-1.

22. Programming Interface – Parallel Mode

Every code byte in the Flash array can be programmed by using the appropriate combination of control signals. The write operation cycle is self-timed and once initiated, will automatically time itself to completion. Most major worldwide programming vendors offer support for the Atmel AT89 microcontroller series. Please contact your local programming vendor for the appropriate software revision

Notes:

1. Each PROG pulse is 200 ns - 500 ns for Chip Erase.

2. Each PROG pulse is 200 ns - 500 ns for Write Code Data.

3. Each PROG pulse is 200 ns - 500 ns for Write Lock Bits.

4. RDY/BSY signal is output on P3.0 during programming.

5.X=don’t Care

Note:

- B1 = 0, B2 = 0 ---> Mode 1, no lock protection B1 = 0, B2 = 1 --->

- Mode 2, lock bit 1 activated B1 = 1, B2 = 0 --->

- Mode 3, lock bit 2 activated B1 = 1, B2 = 1 --->

- Mode 4, lock bit 3 activated After Reset signal is high, SCK should be low for at least 64 system clocks before it goes high to clock in the enable data bytes. No pulsing of Reset signal is necessary. SCK should be no faster than 1/16 of the system clock at XTAL1. For Page Read/Write, the data always starts from byte 0 to 255. After the command byte and upper address byte are latched, each byte thereafter is treated as data until all 256 bytes are shifted in/out. Then the next instruction will be ready to be decoded.

LCD Display

LCD (liquid crystal display) screen is an electronic display module and finds a wide range of applications. A 16 cross 2 LCD is a very basic module and is very commonly used in various devices and circuits. These modules are preferred over seven segment displays and other multi segment LEDs. The reason being : LCDs are economical, easily programmable, have no limitation of displaying special and even custom characters(unlike in seven segments),animations and so on. A  16x2 LCD means it can display 16 characters per line and there are two such lines. In this LCD each character is displayed in 5x7 pixel matrix. The LCD has two registers, namely, Command and Data.

The Command register stores the command instructions given to the LCD. A command is an instruction given LCD to do predefined tasks like initialising it, clearing its screen, setting the cursor position, controlling display etc. The data register stores the data to be displayed on the LCD. The data is in the ASCII value of the character to be displayed on the LCD screen.

HITACHI HD 44780

The LCD display that we have used in our project is the HITACHI HD44780. It is one of the most common dot matrix liquid crystal displays. Hitachi developed the microcontroller specifically to drive alphanumeric LCD display with a simple interface that could be connected to a general purpose microcontroller or microprocessor. Many manufacturers of displays integrated the controller with their product making it the informal standard for this type of displays. The device can display ASCII characters, Japanese Kana characters, and some symbols in two 28 character lines. Using an extension driver, the device can display up to 80 characters.

Features:

- 5x8 and 5x10 dot matrix possible

- Low power operation support (2.7 to 5.5V)

- Wide range of liquid crystal display driver power(3 to 11V)

- Liquid Crystal drive waveform

- Correspond to high MPU bus interface(2MHz when Vcc =5V)

- 4bit or 8bit MPU interface enabled.

- 80x8bit display RAM(80 characters max)

- 64x8 bit character generator RAM.

- 16 common X 40 segment liquid crystal display driver

- Wide range of instruction functions:

Display clear, cursor home, display ON/OFF, display character blink, cursor shift, and display shift.

- Pin function compatibility with HD44780S

- Internal oscillator with external resistors

- Low power consumption

Development  and  Implementation

Our project has come a long way in terms of development from its very early stages at the beginning of our final academic year to our finalised element today.

The basic idea of our project was to create and implement a flexible security system using microcontroller as the major element. We had a pool of opinions about implementing our concept from where we had to select one which seemed both ambitious and at the same time feasible. Eventually after a long debate our project mentor Professor Arunava Mukherjee finally gave us the idea of using a security door lock system which to us seemed very challenging as well as achievable. So we decided on making a password protected door lock system using the microcontroller.

The microcontroller being the primary element of our project, demanded the highest priority in terms of selection. We decided to go with AT89S52(ATMEL)as our microcontroller.

The AT89S52 microcontroller

The AT89S52 is a low-power, high-performance CMOS 8-bit microcontroller with 8KB of ISP flash memory. The device uses Atmel high-density, non-volatile memory technology and is compatible with the industry-standard 80C51 instruction set and pin out. On-chip flash allows the program memory to be reprogrammed in-system or by a conventional non-volatile memory programmer. This powerful microcontroller is suitable for many embedded control applications.

As mentioned above, there were several developments made to our project since we first started. These developments are illustrated below.

- Firstly, talking about the coding part- the primary idea was to have a predefined password in the microcontroller program. This password was added to the program and the user needed to know this password to unlock the system that is the door. Everything was falling into order until we encountered one large setback. The term flexibility was completely missing from our system. To be more precise, once the password was set, there was no way of changing it under any circumstances. The whole system needed to be reprogrammed for achieving this flexibility.

Something had to be done to overcome this problem and once again our mentor Arunava sir came up with a solution. Some major alterations were done in the programming part (by Sagnik Hajra) to overcome this drawback. The program was developed to have a master password which would be provided initially with the microcontroller chip. This password was only available to the administrator of the security system. To start up the system, the administrator required to provide the master password, following this step the administrator could set up their desired password. So the door then opened only after typing in the password set by the administrator.

In case the password needed to be changed, the administrator could RESET the system using an additional RESET button that was provided as a part of the development, then setting the new desired password.

This turned out to be a mammoth leap in terms of flexibility as the password changing was no longer an issue. The password changing procedure then transformed into an user friendly practise from a baffling dilemma.

- The next big development in our project was regarding the keypad. Initially it was decided that we would be using an 8 switch keypad but it turned out as a failure due it a mismatch of synchronisation. The problem was that on pressing a single key, all the other keys were being connected virtually. To overcome this predicament we attempted using a 16 key keypad, but this keypad too was no different. The same problem still persisted.

The solution came in the form of 4x4 matrix keypad. This keypad also had 16 keys. All four rows were internally connected to a single port from where another connection was taken to the port-1 of the microcontroller. Port 1.0-1.3 was used to connect the four rows to the microcontroller. Similarly the columns were connected to port-2 of the microcontroller (port 2.0-2.3).

Some adjustments were made to the program to adapt to these changes and for proper functioning of the keypad. This change finally got rid of the problem we faced.

- Another part that was missing from our project was a display system, the password was entered but there was no display- this seemed incomplete.

This led to the next big development in our project, we added a LCD display to our system. The LCD was connected to an empty port (port-0) of the microcontroller. The LCD microcontroller we used was HITACHI HD44782 and it was programmed to display the following:

- WELCOME- when the right password was pressed.

- WARNING- when a wrong password was entered.

- PROHIBITED- when a wrong password was entered three times at a stretch.

The key feature of this LCD display is given in details in the LCD part of our project documents. This new addition to our endeavour seemed to fit in just fine as it gave the user a much more amicable interface to work with. Furthermore the display gave the user a sense of security as he/she had a full exposure to the system. To be precise the user could actually see that the password was correct or incorrect. But most importantly the LCD display helped in starting out the system, specially during feeding in the master password and setting the desired password as per the first development in our project (coding part).

- The last piece of improvement that we did was the addition of an electromechanical system to replicate a door. This idea was given to us by professorArunava Mukherjee. We used a gear motor as the primary mechanism for opening the door. The motor was connected to the microcontroller and the door in turn was hooked to the motor. Once the correct password was given, a potential difference was applied to the motor which resulted in opening the door. The door remained open for a predefined time and then after a delay, it closed. This was achieved by applying the same voltage across the microcontroller but in a reverse polarity. To ensure this proper functioning of this mechanism, some alterations and additions was done to the microcontroller program.

These were the mentionable developments that our project went through in the stretch of our final academic year.

THREE MAIN PARTS OF PROJECT

A) CIRCUIT DESIGN

Col2

Col4

Col3

Col 1

Row 4

Row 3

Row 2

LCD

Row 1

AT89S52

Pressed

EXPLAINATION

The above circuit is comprised of the following components:

The primary component of the circuit design is the microcontroller AT89S52.

It contains 40 pins which are connected to several other components as shown above.

- The circuit diagram contains a 4x4 matrix keypad. The 4 rows of this keypad is connected to port 1 (1.0-1.3) of the microcontroller.

Similarly, the 4 columns are connected to port 2 (2.0-2.3) of the microcontroller.

- The next component of the circuit is the LCD display which is connected to port 0 of the microcontroller. The LCD has 8 8bit data pins which are  connected to port 0 (0.0-0.7) of the microcontroller. The first five pins of the LCD is GND, Vcc, Vee, Rs, RW and EN respectively.

- Pin number 9 is the RESET button which is grounded via a 8.2K ohm resistor and a 1microF capacitor. This is the RESET button which is needed to reset the whole system.

- A 12MHz oscillator is connected to the microcontroller at pin 18 and 19. The main function of the oscillator is that it is acts a clock input and determines the operating frequency of the microcontroller. Two 33pF capacitors are connected to the oscillator as a safety measure. This ensures the safety of the oscillator in case it is subjected to high voltage or a surge.

- There are four switches connected to port 3 (3.1, 3.3, 3.5, 3.6) of the microcontroller. They are namely ON, RESET, ENTER and DOOR OPEN.

- Pin 20 is GROUND and pin 40 is Vcc in the microcontroller.

- Pin 28, 29, 30 are connected to the EN, R/W, RS pins of the LCD respectively.

Check Keypad

START

If ‘Door open from inside’ is pressed

Scans Keypad

DATA

FLOW

After Delay

Pressed

Not Pressed

Correct

NO

Yes

Wrong

NO

Yes

NO

Yes

Not Pressed

Correct

Wrong

Password Limit Exceed

Warning/ Yellow LED on

Door opens/ green LED on

Door closes/ Green LED off

Checks for the Secret key

Compare  password

Password limitExceed

Checks for the Secret key

Checks if the wrong password is entered 3X

Compare MasterPassword

Password limitExceed

Enter  Master Password

Set Password

Warning/ Yellow LED on

Input password

Alarm On/ Red LED on

Alarm On/ Red LED on

- SOURCE CODE/PROGRAM

.ORG 0000H

CORRECT:MOV P3,#0FFH

MOV 20H,#0FFH

MOV 21H,#0FFH

MOV 22H,#0FFH

MOV 23H,#0FFH

MOV 24H,#0FFH

MOV 25H,#0FFH

MOV 26H,#0FFH

MOV 27H,#0FFH

MOV 28H,#0FFH

MOV 29H,#0FFH

MOV 2AH,#0FFH

MOV 2BH,#0FFH

MOV 2CH,#0FFH

MOV 2DH,#0FFH

MOV 2EH,#0FFH

MOV 2FH,#0FFH

MOV R0,#20H

MOV A,#0FFH

MOV A,#38H

LCALL COMMAND

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

MOV A,0EH

LCALL COMMAND

LCALL DELAY

MOV A,80H

LCALL COMMAND

SWITCH:LCALL DELAY

MOV P1,#0

MOV P2,#0FFH

MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,SWITCH

MOV P3,#0FFH

OVER01:JB P3.6,K1   ;ENTER SWITCH

LJMP RCOMP

K1:CJNE R0,#32H,GO_TO

LCALL PASSWORD_EXCEED

LJMP LOOP0

GO_TO:MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,OVER02

SJMP OVER01

OVER02:LCALL KEY_BOARD

SJMP SWITCH

RCOMP:MOV A,20H

CJNE A,#0EEH,WARNING

MOV A,21H

CJNE A,#0EDH,WARNING

MOV A,22H

CJNE A,#0EBH,WARNING

MOV A,23H

CJNE A,#0E7H,WARNING

MOV A,24H

CJNE A,#0FFH,WARNING

SJMP NEXT_STEP

WARNING:CLR P3.4;RED LIGHT

LOOP0:JB P3.5,LOOP0

LJMP CORRECT

NEXT_STEP:LCALL SET_THE_PASSWORD

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

MOV 50H,#0FFH

MOV 51H,#0FFH

MOV 52H,#0FFH

MOV 53H,#0FFH

MOV 54H,#0FFH

MOV 55H,#0FFH

MOV 56H,#0FFH

MOV 57H,#0FFH

MOV 58H,#0FFH

MOV 59H,#0FFH

MOV 5AH,#0FFH

MOV 5BH,#0FFH

MOV 5CH,#0FFH

MOV 5DH,#0FFH

MOV 5EH,#0FFH

MOV 5FH,#0FFH

MOV P3,#0FFH

MOV R0,#50H

MOV A,#38H

LCALL COMMAND

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

MOV A,0EH

LCALL COMMAND

LCALL DELAY

MOV A,80H

LCALL COMMAND

SET_PASSWORD:LCALL DELAY

MOV P1,#0

MOV P2,#0FFH

MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,SET_PASSWORD

MOV P3,#0FFH

OVER11:JB P3.6,K11

LJMP START

K11:CJNE R0,#5FH,GO_TO1

SJMP MESSAGE

GO_TO1:MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,OVER12

SJMP OVER11

OVER12:LCALL KEY_BOARD

SJMP SET_PASSWORD

MESSAGE:CLR P3.2

LCALL PASSWORD_EXCEED

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LJMP NEXT_STEP

START:MOV R4,#04H

MOV P3,#0FFH

HERE:JB P3.3,HERE1  ;ON SWITCH

SJMP AGAIN

HERE1:JB P3.1,HERE   ;RESET PASSWORD SWITCH

LJMP CORRECT

AGAIN:MOV 20H,#0FFH

MOV 21H,#0FFH

MOV 22H,#0FFH

MOV 23H,#0FFH

MOV 24H,#0FFH

MOV 25H,#0FFH

MOV 26H,#0FFH

MOV 27H,#0FFH

MOV 28H,#0FFH

MOV 29H,#0FFH

MOV 2AH,#0FFH

MOV 2BH,#0FFH

MOV 2CH,#0FFH

MOV 2DH,#0FFH

MOV 2EH,#0FFH

MOV 2FH,#0FFH

MOV R0,#20H

MOV A,#0FFH

MOV A,#38H

LCALL COMMAND

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

MOV A,0EH

LCALL COMMAND

LCALL DELAY

MOV A,80H

LCALL COMMAND

SWITCH11:LCALL DELAY

MOV P1,#0

MOV P2,#0FFH

MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,SWITCH11

MOV P3,#0FFH

OVER21:JB P3.6,K111

LJMP COMP

K111:CJNE R0,#32H,GO_TO11

LCALL PASSWORD_EXCEED

LJMP LOOP

GO_TO11:MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,OVER22

SJMP OVER21

OVER22:LCALL KEY_BOARD

SJMP SWITCH11

COMP:MOV A,20H

CJNE A,50H,WARN

MOV A,21H

CJNE A,51H,WARN

MOV A,22H

CJNE A,52H,WARN

MOV A,23H

CJNE A,53H,WARN

MOV A,24H

CJNE A,54H,WARN

MOV A,25H

CJNE A,55H,WARN

MOV A,26H

CJNE A,56H,WARN

MOV A,27H

CJNE A,57H,WARN

MOV A,28H

CJNE A,58H,WARN

MOV A,29H

CJNE A,59H,WARN

MOV A,2AH

CJNE A,5AH,WARN

MOV A,2BH

CJNE A,5BH,WARN

MOV A,2CH

CJNE A,5CH,WARN

MOV A,2DH

CJNE A,5DH,WARN

MOV A,2EH

CJNE A,5EH,WARN

MOV A,2FH

CJNE A,5FH,WARN

CLR P3.0  ;GREEN LIGHT

LCALL WELCOME

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LJMP START

WARN:DJNZ R4,YELLOW

SJMP LOOP

YELLOW:CLR P3.2;YELLOW LIGHT

LCALL SHOW_WARNING

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

LCALL DELAY

SETB P3.2

LJMP AGAIN

LOOP1:JB P3.5,LOOP1

LJMP START

LOOP:CLR P3.4

LCALL PROHIBITED

SJMP LOOP1

KEY_BOARD:LCALL STAR

MOV P1,#11111110B

MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,ROW_0

MOV P1,#11111101B

MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,ROW_1

MOV P1,#11111011B

MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,ROW_2

MOV P1,#11110111B

MOV A,P2

ANL A,#00001111B

CJNE A,#00001111B,ROW_3

RET

ROW_0:ORL A,#11100000B

MOV @R0,A

INC R0

RET

ROW_1:ORL A,#11010000B

MOV @R0,A

INC R0

RET

ROW_2:ORL A,#10110000B

MOV @R0,A

INC R0

RET

ROW_3:ORL A,#01110000B

MOV @R0,A

INC R0

RET

STAR:MOV A,#'*'

LCALL DATA

RET

SET_THE_PASSWORD:MOV A,#38H

LCALL COMMAND

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

MOV A,0EH

LCALL COMMAND

LCALL DELAY

MOV A,80H

LCALL COMMAND

LCALL DELAY

MOV A,#'S'

LCALL DATA

LCALL DELAY

MOV A,#'E'

LCALL DATA

LCALL DELAY

MOV A,#'T'

LCALL DATA

LCALL DELAY

MOV A,#' '

LCALL DATA

LCALL DELAY

MOV A,#'P'

LCALL DATA

LCALL DELAY

MOV A,#'A'

LCALL DATA

LCALL DELAY

MOV A,#'S'

LCALL DATA

LCALL DELAY

MOV A,#'S'

LCALL DATA

LCALL DELAY

MOV A,#'W'

LCALL DATA

LCALL DELAY

MOV A,#'O'

LCALL DATA

LCALL DELAY

MOV A,#'R'

PASSWORD_EXCEED:MOV A,#38H

LCALL COMMAND

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

MOV A,#0EH

LCALL COMMAND

LCALL DELAY

MOV A,#80H

LCALL COMMAND

LCALL DELAY

MOV A,#'P'

LCALL DATA

LCALL DELAY

MOV A,#'A'

LCALL DATA

LCALL DELAY

MOV A,#'S'

LCALL DATA

LCALL DELAY

MOV A,#'S'

LCALL DATA

LCALL DELAY

MOV A,#'W'

LCALL DATA

LCALL DELAY

MOV A,#'O'

LCALL DATA

LCALL DELAY

MOV A,#'R'

LCALL DATA

LCALL DELAY

MOV A,#'D'

LCALL DATA

LCALL DELAY

MOV A,#' '

LCALL DATA

LCALL DELAY

MOV A,#'L'

LCALL DATA

LCALL DELAY

MOV A,#'I'

LCALL DATA

LCALL DELAY

MOV A,#'M'

LCALL DATA

LCALL DELAY

MOV A,#'I'

LCALL DATA

LCALL DELAY

MOV A,#'T'

LCALL DATA

LCALL DELAY

MOV A,#' '

LCALL DATA

LCALL DELAY

MOV A,#'E'

LCALL DATA

LCALL DELAY

MOV A,#'X'

LCALL DATA

LCALL DELAY

MOV A,#'C'

LCALL DATA

LCALL DELAY

MOV A,#'E'

LCALL DATA

LCALL DELAY

MOV A,#'E'

LCALL DATA

LCALL DELAY

MOV A,#'D'

RET

WELCOME:MOV A,#38H

LCALL COMMAND

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

MOV A,#0EH

LCALL COMMAND

LCALL DELAY

MOV A,#06H

LCALL COMMAND

LCALL DELAY

MOV A,#80H

LCALL DATA

LCALL DELAY

MOV A,#'W'

LCALL DATA

LCALL DELAY

MOV A,#'E'

LCALL DATA

LCALL DELAY

MOV A,#'L'

LCALL DATA

LCALL DELAY

MOV A,#'C'

LCALL DATA

LCALL DELAY

MOV A,#'O'

LCALL DATA

LCALL DELAY

MOV A,#'M'

LCALL DATA

LCALL DELAY

MOV A,#'E'

LCALL DATA

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

RET

SHOW_WARNING:MOV A,#38H

LCALL COMMAND

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

MOV A,#0EH

LCALL COMMAND

LCALL DELAY

MOV A,#06H

LCALL COMMAND

LCALL DELAY

MOV A,#80H

LCALL COMMAND

LCALL DELAY

MOV A,#'W'

LCALL DATA

LCALL DELAY

MOV A,#'A'

LCALL DATA

LCALL DELAY

MOV A,#'R'

LCALL DATA

LCALL DELAY

MOV A,#'N'

LCALL DATA

LCALL DELAY

MOV A,#'I'

LCALL DATA

LCALL DELAY

MOV A,#'N'

LCALL DATA

LCALL DELAY

MOV A,#'G'

LCALL DATA

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

RET

PROHIBITED:MOV A,#38H

LCALL COMMAND

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

MOV A,#0EH

LCALL COMMAND

LCALL DELAY

MOV A,#06H

LCALL COMMAND

LCALL DELAY

MOV A,#80H

LCALL COMMAND

LCALL DELAY

MOV A,#'P'

LCALL DATA

LCALL DELAY

MOV A,#'R'

LCALL DATA

LCALL DELAY

MOV A,#'O'

LCALL DATA

LCALL DELAY

MOV A,#'H'

LCALL DATA

LCALL DELAY

MOV A,#'I'

LCALL DATA

LCALL DELAY

MOV A,#'B'

LCALL DATA

LCALL DELAY

MOV A,#'I'

LCALL DATA

LCALL DELAY

MOV A,#'T'

LCALL DATA

LCALL DELAY

MOV A,#'E'

LCALL DATA

LCALL DELAY

MOV A,#'D'

LCALL DATA

LCALL DELAY

MOV A,#01H

LCALL COMMAND

LCALL DELAY

RET

COMMAND:MOV P1,A

CLR P2.4;RS

CLR P2.5;RW

SETB P2.6;CE

CLR P2.6;CE

RET

DATA:MOV P1,A

SETB P2.4;RS HIGH

CLR P2.5;RW LOW

SETB P2.6; ENABLE HIGH

CLR P2.6; ENABLE LOW

RET

DELAY:MOV R7,#01H

LX3:MOV R6,#05H

LX2:MOV R5,#0AH

LX1:DJNZ R5,LX1

DJNZ R6,LX2

DJNZ R7,LX3

RET

.END

- DOOR LOCK SYSTEM

Apart from the microcontroller part there is mechanical part in the device which is concerned with opening the door. When the correct password is entered, the door will be opened automatically and will be closed after a while.

A geared motor with a groove bar is used to open the door. One terminal of motor are connected with the pin of microcontroller and other with the voltage source. The microcontroller passes a signal to generate a voltage difference between the terminals of motor and it rotates to open the door. A delay is set in the program till the door is opened. After the delay polarity is changed to rotate the motor in reverse direction to shut the door.

Applications

The term access control refers to the practice of restricting entrance to a property, a building or a room to authorized persons. Integrated and controller based access control numeric and alphabetic keyless entry keypads with programmable features for the access control of single openings. The various applications are as follows.

• Industrial facilities

• Commercial buildings

• Airports, civil transportation

• Convention halls

• Psychiatric care centres

• Museums and fine art galleries

• Ware houses

• Technology centres

• Government and military

• Long term care facilities

• Drug rehabilitation centres

Advantages

One of the main advantages of a keyless door lock is the fact that you will not be required to carry a key. This makes it less likely that you'll lose the key and have a difficult time getting in to the home. It also means that you won't need to store a spare key somewhere on your property, as many home owners currently do. This poses a security risk, so eliminating the need for that additional key helps to make your home safer.

Another advantage of a keyless door lock is the fact that you can provide entry into your home to others at your discretion. You will not need to make copies of a key or leave keys for someone else to get in to your home. Rather, you'll simply need to tell those people the proper code to put into the keyless entry system in order to gain access. With the system in place, you can also tell exactly when and how people attempted to access your home. This gives you a good idea of how secure your house is.

Disadvantages

While there are many benefits to including a keyless door lock system in your home, there are also some major drawbacks to consider as well. The first of these is a security issue. While keyless door lock systems are generally very safe and are designed to alert police or other authorities if incorrect codes are entered too many times, it is nonetheless possible that an intruder may be able to gain access to your home through this system by guessing or hacking the code.

Just as you can forget your keys and be locked out of your home, you can also forget the pass code to access your keyless entry system and be locked out as well. This can pose a problem if you have a difficult time remembering things; it's safest to use a completely random code and to avoid things like birth dates.

One final disadvantage of keyless door locks is that those systems which are powered by electricity may not function properly in the case of a power failure. This can leave your door completely locked throughout the failure, or it may result in the door not locking properly and remaining open during the failure as well.

Problem faced during development and solutions

- We faced the problem regarding connecting the keyboard to the microcontroller.

This problem was resolved by using matrix keyboard.

- The programming of microcontroller was not working initially,but was dealt with later.

- Difficulties were faced during the implementation for several functionalities for the user.

- Circuit design was delicate and its minute details had to be taken care of, which was tedious and time taking.

- Faced problem while implementation of interface between keyboard, circuit and the LCD display.

Bibliography

REFERENCE BOOKS:

• Programming in ANSI C: E BALAGURUSAMY

• The 8051microcontroller and embedded systems: MUHAMMAD ALI MAZIDI

JANICE GILLISPIE MAZIDI

• The 8051 microcontroller: KENNETH J. AYALA

􀂙WEB SITES:

• www.atmel.com

• www.national.com

• www.google.com

• www.scientech.bz

ANNEXTURES

Annexture I

Manual FOR THE ADMINISTRATOR

- A master password is provided with the product which you need to keep secret and it cannot be changed.

- Initially you have to enter the master password correctly to set the password for the device.

- Whenever you need to change the password, master password is required. So only you have the authority to change the password.

- When alarm will be activated due to entering the wrong password, you have to press reset button to stop the alarm.

- There should be 24X7 power supply to the system. The circuit will not work in the absence of power supply.

- If Power supply goes off, the program will come to its initial state where the administrator need to set the password again with the help of its Master password.